EEE 3310 Digital Electronics | Fall 2022

# Design Project 2

Design & Simulation of Digital Circuits with CAD Tools

Total Points: 150

# Due: 7 December 2022

Design, simulate, and analyze the following logic gates using HSPICE. Use the 45-nm CMOS High Performance Predictive Technolog Model. Supply voltage, VDD, should be 1V for all the designs.

- 2-input NAND gate

- 2-input NOR gate

- 2-input XNOR gate

Note: you will need to perform transient analysis for this. Apply a pulse at the input (VI). Use the following pulse as input: PULSE (0 1 0n 0.01n 0.01n 4n 8n). Use .measure to

measure the tPLH and tPHL values.

| Performance Parameters | Basic CMOS Inverter |
| --- | --- |
| tPLH | 15.97ps |
| tPHL | 17.25ps |
| tp | 16.61ps |

# Part 1: 2-input NAND gate design, simulation, and analysis (40 points)

- Determine the widths (W) of all the transistors in the design such that the delays are almost same as that of the basic inverter in Part 0.

# Include the diagram of the 2-input CMOS NAND logic clearly indicating the (W/L) of each transistor. Note: the diagram should NOT be hand-drawn.

2n=4, Wn=180

p=3, Wp=135

2n=4, Wn=180

p=3, Wp=135

- Model the circuit in HSPICE, perform the HSPICE simulation, and note down the following parameters.

Note: you will need to perform transient analysis for this. Apply two pulses at the inputs A and

B. One pulse should have a time period of 8ns, whereas the other should have a period of 16ns. Use .measure to measure the values based on the transient response.

| Performance Parameters | 2-input NAND gate |  |
| --- | --- | --- |
| tPLH | 17.8, 17.3, 9.66 | 14.92ps |
| tPHL | 14.7, 15.4, 16.7 | 15.6ps |
| tp | 15.26ps |  |
| Static Power | 25.9575nW |  |
| Average Power | 218.407nW |  |

Include the screenshot of the transient response waveform to observe the inverter operation up to 20ns and the SPICE netist (.sp file).

- Is the average propagation delay,tp ,exactly same as that of the basic CMOS inverter from Part 0? If not, what is the percentage deviation from the basic inverter’s avergae propagation delay?

8.127%

# Part 2: 2-input NOR gate design, simulation, and analysis (40 points)

- Determine the widths (W) of all the transistors in the design such that the delays are almost same as that of the basic inverter in Part 0.

# Include the diagram of the 2-input CMOS NOR logic clearly indicating the (W/L) of each transistor. Note: the diagram should NOT be hand-drawn.

n=2, Wn=90

n=2, Wn=90

2p=6, Wp=270

2p=6, Wp=270

- Model the circuit in HSPICE, perform the HSPICE simulation, and note down the following parameters.

| Performance Parameters | 2-input NOR gate |  |
| --- | --- | --- |
| tPLH | 16.8, 15.6, 17.7 | 16.433ps |
| tPHL | 20.4, 10.4, 18.5 | 16.7ps |
| tp | 16.566ps |  |
| Static Power | 4.1915nW |  |
| Average Power | 89.04nW |  |

Include the screenshot of the transient response waveform to observe the inverter operation up to 20ns and the SPICE netist (.sp file).

- Is the average propagation delay,𝒕𝒕𝒑𝒑 ,exactly same as that of the basic CMOS inverter from Part 0? If not, what is the percentage deviation from the basic inverter’s avergae propagation delay?

0.260%

# Part 3: 2-input XNOR gate design, simulation, and analysis (50 points)

- Determine the widths (W) of all the transistors in the design such that the delays are almost same as that of the basic inverter in Part 0.

# Include the diagram of the 2-input CMOS XNOR logic clearly indicating the (W/L) of each transistor. Note: the diagram should NOT be hand-drawn.

2p=6, Wp=270

n=2, Wn=90

p=3, Wp=135

VDD

B

B’

A’

2p=6, Wp=270

P4

P3

P2

P1

2n=4, Wn=180

2n=4, Wn=180

2n=4, Wn=180

2n=4, Wn=180

2p=6, Wp=270

2p=6, Wp=270

gnd

- Model the circuit in HSPICE, perform the HSPICE simulation, and note down the following parameters.

| Performance Parameters | 2-input XNOR gate |  |
| --- | --- | --- |
| tPLH | 15.6, 22.6, 16, 24.5, | 19.675ps |
| tPHL | 25.6, 19.4, 15.6, 22.8 | 20.85ps |
| tp | 20.262ps |  |
| Static Power | 390.9029nW |  |
| Average Power | 255.84nW |  |

Include the screenshot of the transient response waveform to observe the inverter operation up to 20ns and the SPICE netist (.sp file).

- Is the average propagation delay,𝒕𝒕𝒑𝒑 ,exactly same as that of the basic CMOS inverter from Part 0? If not, what is the percentage deviation from the basic inverter’s avergae propagation delay?

21.989%

Deliverables:

One PDF Report with all the designs, .SP files, simulation results, question answers, waveform snippets, and tables. (20 points)

CMOS Inverter Part 0

* This is the model for a 45-nm CMOS Inverter Part 0*

.OPTION POST

.include 'W:\Digital Electronics\device_45_nm.lib'

.param len= 45nm

.param Wp= 135nm

.param Wn= 90nm

vdd vdd gnd dc 1V

*FOR TRANSIENT ANALYSIS

*PULSE (V1 V2 	Td  Tr  Tf  PW  Period)

V1 Vin gnd PULSE (0 1 0n 0.01n 0.01n 4n 8n)

* TRANSISTOR CONNECTION

*UDN	Drain	Gate 	Source	Sub	device_type	Width	Length

M2 	Vout 	Vin 	vdd 	vdd	pmos 		W= Wp 	L= len

M1 	Vout 	Vin 	gnd 	gnd 	nmos 		W= Wn 	L= len

CL Vout gnd 3f

* MEASURING DELAY IN TRANSIENT ANALYSIS

.tran 20p 50n

.measure tpLH TRIG V(Vin) VAL=0.5V FALL=1 TARG V(Vout) VAL=0.5V RISE=1

.measure tpHL TRIG V(Vin) VAL=0.5V RISE=2 TARG V(Vout) VAL=0.5V FALL=2

.measure tp PARAM = '(tpLH + tpHL)/2'

.PRINT V(Vout) V(Vin)

.end

CMOS 2-input NAND gate Part 1

*This is the model for a 45-nm CMOS 2-input NAND gate Part 1 *

.OPTION POST

.include 'W:\Digital Electronics\device_45_nm.lib'

.param len= 45nm

.param Wp= 135nm

.param Wn= 180nm

vdd vdd gnd dc 1V

*FOR TRANSIENT ANALYSIS

*PULSE (V1 V2 	Td  Tr  Tf  PW  Period)

V1 VA gnd PULSE (1 0 0n 0.01n 0.01n 2n 6n)

V2 VB gnd PULSE (1 0 0n 0.01n 0.01n 6n 10n)

* TRANSISTOR CONNECTION

*UDN	Drain	Gate 	Source	Sub	device_type	Width	Length

MUA 	VY 	VA 	vdd 	vdd	pmos 		W= Wp 	L= len

MUB 	VY 	VB 	vdd 	vdd	pmos 		W= Wp 	L= len

MDA 	VY 	VA 	P1 	P1 	nmos 		W= Wn 	L= len

MDB 	P1 	VB 	gnd 	gnd 	nmos 		W= Wn 	L= len

CL VY gnd 3f

* MEASURING DELAY IN TRANSIENT ANALYSIS

.tran 20p 50n

.measure tpLH TRIG V(VA) VAL=0.5V FALL=1 TARG V(VY) VAL=0.5V RISE=1

.measure tpHL TRIG V(VA) VAL=0.5V RISE=2 TARG V(VY) VAL=0.5V FALL=2

.measure tp PARAM = '(tpLH + tpHL)/2'

.measure TRAN iavg AVG i(vdd) FROM=18e-9 TO=26e-9

.PRINT V(Vout) V(Vin)

.end

CMOS 2-input NOR gate part 2

* This is the model for a 45-nm CMOS 2-input NOR gate part 2*

.OPTION POST

.include 'W:\Digital Electronics\device_45_nm.lib'

.param len= 45nm

.param Wp= 270nm

.param Wn= 90nm

vdd vdd gnd dc 1V

*FOR TRANSIENT ANALYSIS

*PULSE (V1 V2 	Td  Tr  Tf  PW  Period)

V1 VA gnd PULSE (1 0 0n 0.01n 0.01n 4n 6n)

V2 VB gnd PULSE (1 0 0n 0.01n 0.01n 6n 10n)

* TRANSISTOR CONNECTION

*UDN	Drain	Gate 	Source	Sub	device_type	Width	Length

MUA 	P1 	VA 	vdd 	vdd	pmos 		W= Wp 	L= len

MUB 	VY 	VB 	P1 	P1	pmos 		W= Wp 	L= len

MDA 	VY 	VA 	gnd 	gnd 	nmos 		W= Wn 	L= len

MDB 	VY 	VB 	gnd 	gnd 	nmos 		W= Wn 	L= len

CL VY gnd 3f

* MEASURING DELAY IN TRANSIENT ANALYSIS

.tran 20p 50n

.measure tpLH TRIG V(VA) VAL=0.5V FALL=1 TARG V(VY) VAL=0.5V RISE=1

.measure tpHL TRIG V(VA) VAL=0.5V RISE=2 TARG V(VY) VAL=0.5V FALL=2

.measure tp PARAM = '(tpLH + tpHL)/2'

.measure TRAN iavg AVG i(vdd) FROM=17e-9 TO=19e-9

.PRINT V(Vout) V(Vin)

.end

CMOS 2-input XNOR gate part 3

* This is the model for a 45-nm CMOS 2-input XNOR gate part 3*

.OPTION POST

.include 'W:\Digital Electronics\device_45_nm.lib'

.param len= 45nm

.param WIp= 135nm

.param WIn= 90nm

.param Wp= 270nm

.param Wn= 180nm

vdd vdd gnd dc 1V

*FOR TRANSIENT ANALYSIS

*PULSE (V1 V2 	Td  Tr  Tf  PW  Period)

V1 VA gnd PULSE (1 0 0n 0.01n 0.01n 2n 4n)

V2 VB gnd PULSE (1 0 0n 0.01n 0.01n 3n 5n)

* TRANSISTOR CONNECTION

*UDN	Drain	Gate 	Source	Sub	device_type	Width	Length

MIUA 	VIA 	VA 	vdd 	vdd	pmos 		W= WIp 	L= len

M1DA 	VIA 	VA 	gnd 	gnd 	nmos 		W= WIn 	L= len

MIUB 	VIB 	VB 	vdd 	vdd	pmos 		W= WIp 	L= len

M1DB 	VIB 	VB 	gnd 	gnd 	nmos 		W= WIn 	L= len

MUNB 	P1 	VB 	vdd 	vdd	pmos 		W= Wp 	L= len

MUNA 	VY 	VA 	P1 	P1	pmos 		W= Wp 	L= len

MUIB 	P2 	VIB 	vdd 	vdd	pmos 		W= Wp 	L= len

MUIA 	VY 	VIA 	P2 	P2	pmos 		W= Wp 	L= len

MDIA 	VY 	VIA 	P3 	P3 	nmos 		W= Wn 	L= len

MDNB 	P3 	VB 	gnd 	gnd 	nmos 		W= Wn 	L= len

MDNA 	VY 	VA 	P4 	P4 	nmos 		W= Wn 	L= len

MDIB 	P4 	VIB 	gnd 	gnd 	nmos 		W= Wn 	L= len

CL VY gnd 3f

* MEASURING DELAY IN TRANSIENT ANALYSIS

.tran 20p 50n

.measure tpLH TRIG V(VA) VAL=0.5V FALL=1 TARG V(VY) VAL=0.5V RISE=1

.measure tpHL TRIG V(VA) VAL=0.5V RISE=2 TARG V(VY) VAL=0.5V FALL=2

.measure tp PARAM = '(tpLH + tpHL)/2'

.measure TRAN iavg AVG i(vdd) FROM=13e-9 TO=18e-9

.PRINT V(Vout) V(Vin)

.end